Scott Ruen

108 S. Humphrey Cir

Shawano, WI 54166

03/25/2026

Home Run Inspection Service, LLC

626 E Center St

Shawano, WI 54166

Phone: 715-853-9718

Report Index

Cover Page	1

Report Index	2

Inspection Summary	3

General	10

Exterior	11

Roofs	15

Chimneys	16

Garage Exterior	17

Garage	18

Kitchen	20

Bathrooms	21

Interior	23

Attic	24

Electrical	26

Structure	27

Basement/Foundation	27

Plumbing	30

Laundry	32

Water Heater	33

Heating and Cooling	34

Furnace	34

Air Conditioning	35

Space Heater	37

Heating	37

Inspection Summary

Inspection Summary

| Definitions |  |  |
| --- | --- | --- |
| Note: All definitions listed below refer to the property or items listed as inspected on this report at the time of the inspection Functional - Performing its function and its condition is appropriate for its age and/or use Defect - A condition of any component of an improvement that a home inspector determines, on the basis of the home inspector's judgment on the day of an inspection, would significantly impair the health or safety of occupants of a property or that, if not repaired, removed, or replaced, would significantly shorten or adversely affect the expected normal life of the component of the improvement. Further Evaluation - Item is not functioning as intended or appears not to be functioning as intended. Needs further evaluation by a qualified professional in the related field of work. Needs Repair - The condition of the item warrants repair but does not pose a health or safety concern nor rise to the level of Defect. Maintenance - Maintenance of item is recommended to prevent premature failure or to maintain its functionality. Monitor - Currently functioning but condition and/or age indicates that limited remaining life is expected. Client is advised to budget for replacement or upgrade. Not Inspected - Item was unable to be inspected for safety reasons, lack of power, inaccessible, not visible, disconnected at time of inspection or was not within the scope of this inspection Not Present - Item not present or not found at time of inspection. |
| Note |  |  |
| This Summary page (or group of pages) is provided for convenience and is not a substitute for reading the entire report and should not be relied upon as the complete list for the client's reference, A home inspector may not report on the market value or marketability of a property or whether a property should or should not be purchased. |
| Noted Defects |  |  |
| Items that have a notable defect according to the definition noted Exterior Driveway/Slab Concrete Cracks: Major cracks from heaving/settling. |
| Potential Safety Hazards |  |  |
| Note: Items considered to be possible safety hazards. Items listed here also fall under the definition of a defect. Exterior Walks/Patio Brick/Pavers/Stone Trip hazard. Electrical Outlets Suggest Ground Fault Circuit Interrupter. See Note on Electrical Section, Required since 1973. GFCI IMAGE. Garage Interior Walls/Ceiling Fire separation missing at wall/ceiling in garage. This is a potential fire hazard. Doors House Door Not fire resistant. Suggest a "20 minute door" rated for use between an attached garage and a house. Garage Door Opener Garage door opener did not mechanically reverse to pressure. This is a potential hazard. Adjust reverse sensitivity. Garage Electrical Suggest Ground Fault Circuit Interrupter protection at all garage outlets. See note on Electrical section, Required since 1978. Garage Attic Fire Blocking Missing fire blocking. This is a potential safety hazard. Suggest consult a carpenter and/or the local code authority. Kitchen Kitchen Sink No air gap device for dishwasher drain line. This can cause dirty water to flow back into the dishwasher. Electrical Suggest Ground Fault Circuit Interrupter protection at all kitchen outlets. See note on Electrical section, Required since 1987. Interior General Areas Smoke Detectors Suggest installing a CO detector on each level. Electrical Main Panel Condition Double tapping of neutral wires. Basement/Foundation Unfinished Basement Areas Electrical Suggest Ground Fault Circuit Interrupter. See note on Electrical section, Required since 1990. Plumbing Gas Piping Condition Recommend bonding gas to water line (this can be done by the water heater) Water Heater Water Heater Condition The water heater relief valve overflow pipe needs to be extended to within 6 inches of floor. This is potential hazard and needs repair. |

| Deferred Cost Items |  |  |
| --- | --- | --- |
| Items that may need repair or replacement in the next 5 years: Air Conditioning Air Conditioner Age Approximately 26 years old. Heating Age Gas Space Heater >20 years. |
| Important Maintenance Items |  |  |
| Note: Items listed are identified as needing repair, further evaluation, maintenance, or monitoring. Exterior Siding Hardboard Damage noted at bottom row, southwest corner. Maintenance is very important to preserve hardboard siding. It is critical to maintain paint and caulk. Chimneys Chimney Chimney Recommendations Have flues cleaned and evaluated annually. Garage Exterior Siding Condition Deteriorated edges. Recommend scrape and paint. Garage Doors Passage Door Southwest door needs paint and binds at bottom. Interior General Areas Fireplaces Damper not opening all the way. Attic Insulation Condition Suggest adding insulation. Ventilation Condition Suggest adding air baffles before adding insulation. Electrical Service Information System Grounded To No water meter jumper present at the time of inspection. Plumbing Utility Sink Sink Cold water handle seized up. |

General

General Information

| Property Inspected |  |  |
| --- | --- | --- |
| 108 S. Humphrey Cir Shawano, WI 54166 |
| Date Of Inspection |  |  |
| 03/25/2026 |
| Client |  |  |
| Scott Ruen |
| Real Estate Agent |  |  |
| Listing Broker |  |  |
| Jim Leisner |
| Inspector |  |  |
| Home Run Inspection Service, LLC 715-853-9718 |
| Attendees |  |  |
| Client. |

Hazardous Materials

Determining the presence or absence of hazardous materials such as lead or asbestos is beyond the scope of this inspection.

Building and Site Information

| Building Faces |  |  |
| --- | --- | --- |
| North. Though the building may or may not face directly North, references to direction in this inspection report assume that the front entrance of the building faces North. |
| Building Type |  |  |
| Single family. |
| Building Occupancy |  |  |
| Occupied, seller was at home. |
| Weather Conditions |  |  |
| Temperature: 32° F to 65° F. |

Please make special note that this report is provided as a home inspection with specific limitations as defined in the Inspection Agreement. This is not an engineering report.

Exterior

Siding

| Hardboard |  |  |
| --- | --- | --- |
| Damage noted at bottom row, southwest corner. Maintenance is very important to preserve hardboard siding. It is critical to maintain paint and caulk. |

Windows/Doors

| Condition |  |  |
| --- | --- | --- |
| Appear undamaged. |

Overhangs/Trim

| Condition |  |  |
| --- | --- | --- |
| Appear functional, Appear undamaged. |

Walks/Patio

| Brick/Pavers/Stone |  |  |
| --- | --- | --- |
| Trip hazard. |

Driveway/Slab

| Concrete |  |  |
| --- | --- | --- |
| Cracks: Major cracks from heaving/settling. |

Grading

| Grading |  |  |
| --- | --- | --- |
| Low or flat areas: Various locations. |

Concrete Porch/Stoop

| Limitations |  |  |
| --- | --- | --- |
| Carpeting restricts inspection. |

| Location |  |  |
| --- | --- | --- |
| Front. |
| Condition |  |  |
| Appears functional, Appears undamaged. |

Concrete Porch/Stoop

| Location |  |  |
| --- | --- | --- |
| Rear. |

| Condition |  |  |
| --- | --- | --- |
| Appears functional, Appears undamaged. |

Exterior Faucet(s)

| Exterior Faucet(s) |  |  |
| --- | --- | --- |
| No flow at one or more faucets. The water supply to the faucet(s) may be turned off inside. Testing shut off valves is beyond the scope of this inspection. |

Electrical

| Outlets |  |  |
| --- | --- | --- |
| Suggest Ground Fault Circuit Interrupter. See Note on Electrical Section, Required since 1973. GFCI IMAGE.. |
| Exterior Lighting Fixtures |  |  |
| Outside light did not turn on. Photo eye may be off. Appear undamaged. |
| Power Feed |  |  |
| Overhead, Appears functional, Appears undamaged. |

Sump Discharge

Appears undamaged, Sump pump discharge extends underground. Monitor for freezing.

Downspouts

Metal, Discharges above grade, Appears undamaged.

Gutters: View From Ground

Appear undamaged.

Roofs

Inspection Limitations

| Inspection Limitations |  |  |
| --- | --- | --- |
| Height. |
| Roof Inspected By |  |  |
| Binoculars, From the ground. Metal roofs are not walked on. |

Pitched Roof

| Material |  |  |
| --- | --- | --- |
| Metal. |
| Layers |  |  |
| Unknown, One layer observed. |
| Condition |  |  |
| Appears functional, Appears undamaged. |
| Recommendations |  |  |
| Monitor exposed fasteners. |

Flashings

| Plumbing Stack |  |  |
| --- | --- | --- |
| Appears undamaged. |

Gutters: View From Roof

Roof not accessed.

Pitched Garage Roof

| Garage Roof |  |  |
| --- | --- | --- |
| Same material as and similar condition to house. |

Garage Gutters: View From Roof

Appear undamaged. Viewed from upper level windows.

Chimneys

Chimney

| Location |  |  |
| --- | --- | --- |
| Two flues located in one chimney structure. |
| Chimney Inspected By |  |  |
| Binoculars, From ground. |
| Type of Chimney |  |  |
| Masonry. |
| Chimney Cap |  |  |
| Top of concrete cap not visible. |
| Flue Type |  |  |
| Tile. |
| Flue Condition |  |  |
| Interior not inspected. |
| Chimney Structure |  |  |
| Appears functional, Appears undamaged. |
| Rain Cover |  |  |
| Appears undamaged. |
| Chimney Recommendations |  |  |
| Have flues cleaned and evaluated annually. |
| Chimney Structure |  |  |
| Appears undamaged. |

| View Up Chimney |  |  |
| --- | --- | --- |
| Restricted view. |

Flashings

| Chimney |  |  |
| --- | --- | --- |
| Metal, Appears functional, Appears undamaged. |

Garage Exterior

Siding

| Type |  |  |
| --- | --- | --- |
| Hardboard. |
| Condition |  |  |
| Deteriorated edges. Recommend scrape and paint. |

Exterior View of Windows/Doors

| Condition |  |  |
| --- | --- | --- |
| Older/worn, Appear undamaged. |

Overhangs/Trim

| Condition |  |  |
| --- | --- | --- |
| Appear functional, Appear undamaged. |

Grading At Garage

| Grading |  |  |
| --- | --- | --- |
| Part or all of ground around the building was not visible at the time of inspection due to snow cover. |

Exterior Electrical

| Outlets |  |  |
| --- | --- | --- |
| None observed. |
| Exterior Lighting Fixtures |  |  |
| Appear functional, Appear undamaged. |

Garage Gutters: View From Ground

Appear functional, Appear undamaged.

Garage Downspouts

| Roof Drainage |  |  |
| --- | --- | --- |
| Appear functional, Appears undamaged. |

Garage

Interior Limitations

| Ceiling |  |  |
| --- | --- | --- |
| 100% finished. |
| Walls |  |  |
| 100% finished. |

Type

Attached.

Building Structure

Wood frame, Appear functional, Appears undamaged.

Interior Walls/Ceiling

Appear functional, Appear undamaged, Fire separation missing at wall/ceiling in garage. This is a potential fire hazard.

Windows

Stationary windows appear undamaged.

Doors

| Passage Door |  |  |
| --- | --- | --- |
| Southwest door needs paint and binds at bottom. |
| House Door |  |  |
| Not fire resistant. Suggest a "20 minute door" rated for use between an attached garage and a house. |
| Vehicle Door |  |  |
| Appears functional, Appears undamaged. |

Floor Slab

Appears functional, Appears undamaged.

Garage Door Opener

Did reverse at safety beam, Garage door opener did not mechanically reverse to pressure. This is a potential hazard. Adjust reverse sensitivity.

Garage Electrical

Suggest Ground Fault Circuit Interrupter protection at all garage outlets. See note on Electrical section, Required since 1978.

Foundation

| Concrete Block |  |  |
| --- | --- | --- |
| Appears functional, Appears undamaged. |

Foundation Wall Movement

| Movement |  |  |
| --- | --- | --- |
| No significant movement evident. |

Garage Attic Limitations

| Attic Access |  |  |
| --- | --- | --- |
| Ceiling access hatch: Viewed from hatch. |
| Inspection Limitations |  |  |
| Stored items. |

Garage Attic Insulation

| R-value |  |  |
| --- | --- | --- |
| Approximate "R" value for ceilings/floor: R-11. |
| Insulation Types |  |  |
| Fiberglass. |
| Condition |  |  |
| Uneven insulation levels. |
| Vapor Barrier |  |  |
| None visible. |

Garage Attic Ventilation

| Type |  |  |
| --- | --- | --- |
| Soffit venting observed. |
| Condition |  |  |
| Appears to have sufficient ventilation at the time of inspection. |

Garage Attic Fire Blocking

Missing fire blocking. This is a potential safety hazard. Suggest consult a carpenter and/or the local code authority.

Garage Attic Roof Structure

| Type |  |  |
| --- | --- | --- |
| Trusses, Plywood sheathing. |
| Condition |  |  |
| Appears functional, Structure appears undamaged. |

Kitchen

Kitchen

| Sink |  |  |
| --- | --- | --- |
| Sink, faucet, and piping appear undamaged. No air gap device for dishwasher drain line. This can cause dirty water to flow back into the dishwasher.. |
| Non-professional drain material noted. Monitor for clogging. |

| Garbage Disposal |  |  |
| --- | --- | --- |
| Appears to function properly. |
| Cabinets/Countertops |  |  |
| Appears functional, Appears undamaged. |
| Exhaust Fan |  |  |
| Built into microwave, Recirculating, Appears functional. |
| Heat/Cool Sources |  |  |
| Heat source provided. |
| Stove Utility |  |  |
| Electric. |
| Electrical |  |  |
| Suggest Ground Fault Circuit Interrupter protection at all kitchen outlets. See note on Electrical section, Required since 1987. |

Bathrooms

Central Bathroom

| Location |  |  |
| --- | --- | --- |
| First floor. |
| Toilet |  |  |
| Appears functional, Appears undamaged. |
| Sink |  |  |
| Sink, faucet, and piping appear undamaged. |
| Counter/Cabinet |  |  |
| Appears functional, Appears undamaged. |
| Electrical |  |  |
| Appears functional (GFCI Protected) |
| Ventilation |  |  |
| Window: Appears functional, Appears undamaged, Fan: Appears functional. |
| Heat/Cool Sources |  |  |
| Heat source provided. |

Upstairs Bathroom

| Toilet |  |  |
| --- | --- | --- |
| Appears functional, Appears undamaged. |
| Sink |  |  |
| Sink, faucet, and piping appear undamaged. |

| Counter/Cabinet |  |  |
| --- | --- | --- |
| Appears functional, Appears undamaged. |
| Electrical |  |  |
| Appears functional (GFCI Protected) |
| Ventilation |  |  |
| Window: Appears functional, Appears undamaged, Fan: Appears functional. |
| Heat/Cool Sources |  |  |
| Heat source provided. |
| Tub |  |  |
| Tub, faucet/spout, and drain appear undamaged. |
| Shower Plumbing |  |  |
| Appears to function properly. |
| Shower Enclosure |  |  |
| Appears functional, Appears undamaged. |

Interior

General Areas

| Location |  |  |
| --- | --- | --- |
| General areas. |
| Limitations |  |  |
| Furnishings prevent/limit full inspection. |
| Exterior Doors and Trim |  |  |
| Wood: Appears functional, Appears undamaged. |
| Interior Doors and Trim |  |  |
| Appears functional, Appears undamaged. |
| Windows and Trim |  |  |
| Limited access: shades/blinds/location/plastic film. Double hung, Appear functional, Appear undamaged. |
| Ceiling and Walls |  |  |
| Ceiling and wall framing is not observable due to finishes/insulation. Appears functional, Appears undamaged. |
| Floors |  |  |
| Appears functional, Appears undamaged. |
| Heat/Cool Sources |  |  |
| Appears functional, Appears undamaged. |
| Smoke Detectors |  |  |
| Alarm(s) sounded when test button was pressed. Suggest installing a CO detector on each level. |

| Ceiling Fans |  |  |
| --- | --- | --- |
| Appears functional, Appears undamaged. |
| Stairs/Balconies |  |  |
| Appears functional, Appears undamaged. |
| Fireplaces |  |  |
| Masonry unit in dining area. Damper not opening all the way. |
| Masonry unit in basement. Appears functional, Appears undamaged. |

Attic

Limitations

| Attic Access |  |  |
| --- | --- | --- |
| Ceiling access hatch: Viewed from hatch. |

Insulation

| R-value |  |  |
| --- | --- | --- |
| Approximate "R" value for ceilings/floor: R-11 to R-19. |
| Insulation Type(s) |  |  |
| Not determined (possibly fiberglass) |

| Condition |  |  |
| --- | --- | --- |
| Suggest adding insulation. |
| Vapor Barrier |  |  |
| None visible. Suggest painting ceilings below attic with a vapor retarding primer paint. |

|  | Suggest consulting with an Energy Rater or Home Performance Professional to solve any condensation, heat loss, and comfort problems. |  |
| --- | --- | --- |

Ventilation

| Type |  |  |
| --- | --- | --- |
| Soffit venting observed. |
| Condition |  |  |
| Suggest adding air baffles before adding insulation.. |
| Attic ventilation must be adequate to prevent structural moisture damage, ice dams, mold, and premature roof failure. Ventilation needs change depending on occupancy. |

Exhaust Ducts

| Vent To |  |  |
| --- | --- | --- |
| Upper bathroom fan vents through roof. Main level bathroom fan termination not found. |
| Condition |  |  |
| No adverse issues noted. |

Roof Structure

| Limitations |  |  |
| --- | --- | --- |
| Roof structure not visible: 20% to 30%. |
| Type |  |  |
| Trusses, Plywood sheathing. |
| Condition |  |  |
| Structure appears functional, Structure appears undamaged. |

Electrical

Service Information

| Service Rating |  |  |
| --- | --- | --- |
| Main service rating: 200 amps. |
| Main Box Rating |  |  |
| 200 amps, 240 V, 120 V. |
| Main Breaker/Fuse |  |  |
| 200 amps. |
| Main Service Wire |  |  |
| 200 amps, Aluminum. |
| Wire Protection |  |  |
| Circuit breakers. |
| Branch Wires |  |  |
| Material: Copper, Types of branch wires: Non-metallic cable (Romex) |
| System Grounded To |  |  |
| It appears that the electrical system is grounded to a ground rod. Ground rods are buried in the ground and usually not visible at the time of inspection. Bonded at water pipe. No water meter jumper present at the time of inspection. |

Capacities noted are determined by a visual inspection only. Capacity concerns should be directed to a licensed electrician.

Main Panel

| Location |  |  |
| --- | --- | --- |
| Basement. |

| Condition |  |  |
| --- | --- | --- |
| Appears functional. Double tapping of neutral wires. |

Low voltage wiring is beyond the scope of this inspection.

Finishes restrict inspection of branch circuits.

All electrical concerns are potential safety hazards and should be evaluated/repaired immediately by a licensed electrician.

Structure

Foundation Configuration

Basement: 100%.

Building Structure

| Exterior Walls |  |  |
| --- | --- | --- |
| Finishes restrict inspection. Wood frame. |
| Overhead Beams |  |  |
| Steel, Appears functional, Appears undamaged. |
| Overhead Joists |  |  |
| Wood lumber, Appears functional, Appears undamaged. |
| Subfloor |  |  |
| Plywood, Appears functional, Appears undamaged. |

Basement/Foundation

Inspection is limited to visible areas only.

Inspection Limitations

| Floor |  |  |
| --- | --- | --- |
| Floor not visible due to carpet: 50% to 60%. |
| Walls |  |  |
| Painted foundation walls (old): 100%. |
| Ceiling |  |  |
| Finished ceiling: 50% to 60%. |

Basement Stairs

Appears undamaged.

Foundation Structure

| Type |  |  |
| --- | --- | --- |
| Concrete block. |
| Condition |  |  |
| Appears functional, Appears undamaged. |

| Cracks |  |  |
| --- | --- | --- |
| No cracks evident. |

|  | Cracks may indicate that there is movement in a wall and the wall has technically failed. The most significant cracks are horizontal or vertical wall cracks that go through the block and are combined with inward deformation, bowing, or movement. A basement with a cracked wall may continue to progressively get worse. Sometimes cracked walls remain unchanged indefinitely. |  |
| --- | --- | --- |

Foundation Wall Movement

| Movement |  |  |
| --- | --- | --- |
| No significant movement evident. |

Foundation Settlement

No evident differential settlement present.

Foundation Seepage or Dampness

| Degree |  |  |
| --- | --- | --- |
| None evident at the time of inspection. |

Since about 1920, basements have been designed with drain tile systems to remove water and water pressure. Drain tile systems are not visible for inspection; only the end of the tile is visible at the sump crock or the floor drain palmer valve. Stains and signs of moisture are often due to poor exterior drainage of surface water, but could be due to poor exterior drainage of surface water, but could also indicate a plugged or damaged drain tile system. It is possible the signs of moisture may have been caused by a condition that no longer exists. IT IS IMPOSSIBLE TO PREDICT THE SEVERITY OR FREQUENCY OF MOISTURE PENETRATION ON ONE INSPECTION VISIT.

Leakage or Signs of Leakage

| Water Stains |  |  |
| --- | --- | --- |
| None apparent at the time of inspection. |

Basement Floor

Concrete: Appears functional, Appears undamaged.

Foundation Drainage

| Drain Tiles |  |  |
| --- | --- | --- |
| Present. |
| Sump Pump |  |  |
| Appears to function normally. |

Footings and drain tiles are not visible for inspection.

Unfinished Basement Areas

| Electrical |  |  |
| --- | --- | --- |
| Suggest Ground Fault Circuit Interrupter. See note on Electrical section, Required since 1990.. |

Plumbing

Water Main

| Source |  |  |
| --- | --- | --- |
| Municipal. |
| Location |  |  |
| The main water shut off is located on the North basement wall. |

| Material |  |  |
| --- | --- | --- |
| Copper. |
| Condition |  |  |
| Appears functional, Appears undamaged. |

Main shut-off not tested. Suggest verifying that the main shut-off valve works properly before it is needed in an emergency.

Functional Water Flow

| General Areas |  |  |
| --- | --- | --- |
| Appears adequate. |

Water Pipes

| Material |  |  |
| --- | --- | --- |
| Copper. |
| Condition |  |  |
| Appears functional, Appear undamaged. |

Valves not operated.

Drain Pipes

| Type of System |  |  |
| --- | --- | --- |
| Municipal sewer. |

| Material |  |  |
| --- | --- | --- |
| Steel, Cast iron, Plastic, PVC. |
| Condition |  |  |
| Appears functional, Appears undamaged. |

Plumbing Vent Pipes

| Material |  |  |
| --- | --- | --- |
| Cast iron. |
| Condition |  |  |
| Appears functional, Appears undamaged. |

Gas Piping

| Location of Gas Shut Off |  |  |
| --- | --- | --- |
| At exterior meter and at furnace/water heater. |
| Condition |  |  |
| Appears functional, Appears undamaged. Recommend bonding gas to water line (this can be done by the water heater) |
| Location of Gas Shut Off |  |  |
| The main gas shut off is located outside on the West side of the building. |

Utility Sink

| Location |  |  |
| --- | --- | --- |
| Basement. |

| Sink |  |  |
| --- | --- | --- |
| Cold water handle seized up. |

Laundry

Laundry

| Location |  |  |
| --- | --- | --- |
| First floor closet. |
| Washing Machine Drains To |  |  |
| Washing machine discharges into a drain in the wall. Not tested. Inspection is limited to visible areas only.. |
| Dryer Utility |  |  |
| Electric. |
| Dryer Venting |  |  |
| Suggest replacing foil dryer vent tubing with a metal dryer vent. Foil vents tend to trap lint, which causes dryers to operate less efficiently. Vents to: Vents to outside wall. |
| Electrical |  |  |
| Not accessible. Located behind units. |

Water Heater

Water Heater

| Location |  |  |
| --- | --- | --- |
| Basement. |
| Type |  |  |
| Gas. |
| Age |  |  |
| Approximately 6 years old. |
| Size |  |  |
| Size: 50 gallons. |
| Condition |  |  |
| The water heater relief valve overflow pipe needs to be extended to within 6 inches of floor. This is potential hazard and needs repair. |

Testing of safety devices is beyond the scope of this inspection.

Heating and Cooling

Types of Systems

Air conditioning provides cooling. See the "Air Conditioning" form for more information. Furnace provides heating. See the "Furnace" form for more information. Gas space heater. See the "Gas Space Heater" form for more information.

Furnace

Testing of safety devices is beyond the scope of this inspection.

Forced Air Furnace

| Location |  |  |
| --- | --- | --- |
| Basement. |

| Unit Services |  |  |
| --- | --- | --- |
| All living areas. |
| Brand |  |  |
| Carrier. |
| Age |  |  |
| Approximately 5 years old. |
| Type |  |  |
| High efficiency. |
| Heating Capacity |  |  |
| 100,000 BTU/hr input. |
| Fuel |  |  |
| Gas. |
| Piping/Valves |  |  |
| Appears undamaged. |

| Burners And Flame Observations |  |  |
| --- | --- | --- |
| Appears functional, Flames/burners appeared normal at the time of inspection. |
| Heat Exchanger |  |  |
| Restricted view: less than 5% visible. |
| Carbon Monoxide |  |  |
| Carbon monoxide not detected at duct/supply air. |
| Exhaust Vent |  |  |
| PVC, Appears functional, Appears undamaged. |
| Cabinet |  |  |
| Appears functional, Appears undamaged. |
| Distribution |  |  |
| Distribution method: Forced air fan at heat exchanger. Non-Insulated sheet metal ducts. Appears functional, Appears undamaged. |
| Filter |  |  |
| Disposable, Clean at the time of inspection. |
| Thermostat |  |  |
| Appears functional. |
| System Condition |  |  |
| Operating properly. |

Recommend professional service annually.

Air Conditioning

Air Conditioner

| Location |  |  |
| --- | --- | --- |
| Basement. |
| Unit Services |  |  |
| All living areas. |
| System Type |  |  |
| Split system, Central (ducted) system.. |
| Inspection Limitations |  |  |
| Unit not tested. Outside temperature below 65° in the past 24 hours prohibited safe testing of unit. Testing an A/C unit in cold weather can cause damage to the unit. |
| Brand |  |  |
| Name tag not found. |
| Age |  |  |
| Approximately 26 years old. |
| Cooling Capacity and Efficiency |  |  |
| Not found/determined. |
| Outside Condensing Unit |  |  |
| Appears undamaged. |

| Air Conditioner Fuses |  |  |
| --- | --- | --- |
| Max fuse: 30 amps, Fuse/breaker inside: 30 amps. |
| Indoor Cooling Unit |  |  |
| Unable to view indoor coil. |
| Temperature Drop |  |  |
| Unit not fired. |
| Distribution |  |  |
| Same as Furnace. |
| Filter |  |  |
| Same as Furnace. |
| Thermostat |  |  |
| Appears undamaged. |

Space Heater

Testing of safety devices is beyond the scope of this inspection.

Gas Space Heater

| Carbon Monoxide |  |  |
| --- | --- | --- |
| Not tested. |

Recommend professional service annually.

Heating

Inspection Limitations

| Gas Space Heater |  |  |
| --- | --- | --- |
| Pilot off. Unit not tested. Seller informed that unit works properly. |

Location

| Gas Space Heater |  |  |
| --- | --- | --- |
| Garage. |

Brand

| Gas Space Heater |  |  |
| --- | --- | --- |
| Carrier. |

BTU/hr

| Gas Space Heater |  |  |
| --- | --- | --- |
| Not found. |

Age

| Gas Space Heater |  |  |
| --- | --- | --- |
| >20 years. |

System Condition

| Gas Space Heater |  |  |
| --- | --- | --- |
| Durable construction, Well maintained. |

Fuel

| Gas Space Heater |  |  |
| --- | --- | --- |
| Natural gas. |

Cabinet

| Gas Space Heater |  |  |
| --- | --- | --- |
| Inspected. |

Piping/Valves

| Gas Space Heater |  |  |
| --- | --- | --- |
| Inspected. |

Heat Exchanger

| Gas Space Heater |  |  |
| --- | --- | --- |
| Inspected. |

Burners And Flame Observations

| Gas Space Heater |  |  |
| --- | --- | --- |
| Sealed/limited access to burners. |

Exhaust Vent

| Gas Space Heater |  |  |
| --- | --- | --- |
| Metal vent pipe. Make sure pipe has 2" clearance from combustibles. |

Thermostat

| Gas Space Heater |  |  |
| --- | --- | --- |
| Appears undamaged. |